AI Overview

You can simulate a 4-degree-of-freedom (DOF) robot arm in MATLAB using the Robotic System Toolbox by defining a rigidBodyTree model with Denavit-Hartenberg (DH) parameters.

MathWorks +1

Define DH Parameters

Create a standard DH parameter matrix where each row represents a joint [theta, d, a, alpha]:

- theta: Joint angle (rotation about).

- d: Link offset (translation along).

- a: Link length (translation along).

- alpha: Link twist (rotation about).

YouTube·Chris Annin

For a basic 4-DOF planar or articulated robot (e.g., 3 revolute joints and 1 base/wrist rotation), define your table in MATLAB:

matlab

% DH Parameters [theta d a alpha]

dh_params = [

0,    1.5,  0,    pi/2;  % Joint 1

pi/2, 0,    2.0,  0;     % Joint 2

0,    0,    1.5,  0;     % Joint 3

0,    0,    1.0,  0      % Joint 4

];

Use code with caution.

Build and Visualize the Robot

Use a rigidBodyTree object and loop through the DH parameters to add rigid bodies and joints:

matlab

% Initialize robot rigid body tree

robot = rigidBodyTree('DataFormat','row');

% Define joint types (e.g., all revolute 'revolute')

jointType = {'revolute', 'revolute', 'revolute', 'revolute'};

body = cell(4,1);

joint = cell(4,1);

for i = 1:4

body{i} = rigidBody(['link' num2str(i)]);

joint{i} = joint(['joint' num2str(i)], jointType{i});

% Set DH parameters using transform function

setFixedTransform(joint{i}, dh_params(i,:), 'dh');

body{i}.Joint = joint{i};

if i == 1

addBody(robot, body{i}, robot.BaseName);

else

addBody(robot, body{i}, body{i-1}.Name);

end

end

% Show the 3D visualization

figure;

show(robot);

axis([-3 3 -3 3 0 4]);

title('4-DOF Robot Arm Simulation');

Use code with caution.

Simulate Motion

Animate the robot by passing a vector of joint configurations into the show function:

matlab

% Example joint configuration vector [q1, q2, q3, q4]

q_sample = [pi/4, -pi/6, pi/3, pi/12];

figure;

show(robot, q_sample);

axis([-3 3 -3 3 0 4]);

Use code with caution.

Watch this tutorial on building a manipulator robot using kinematic DH parameters in MATLAB:

20m

Forward Kinematics: Example of 4-DoF SCARA and 6- ...

9.2K views · 4 years ago

YouTube · Arun Dayal Udai

If you'd like, let me know:

- Your specific DH parameter values or robot configuration (e.g., SCARA, articulated)

- Whether you want to perform forward kinematics animation or inverse kinematics trajectory planning

I can tailor the MATLAB script to your exact setup.